Noise Pollution Monitoring

Building the project idea:

#include <LiquidCrystal.h> // include the LiquidCrystal library

const int micPin1 = A0; // define the pin for the first microphone

const int micPin2 = A1; // define the pin for the second microphone

const int micPin3 = A2; // define the pin for the third microphone

const int buzzerPin = 9; // define the pin for the buzzer

const int ledPin = 6; // define the pin for the LED

const int contrast = 50; // define the LCD contrast

LiquidCrystal lcd(12, 11, 5, 4, 3, 2); // initialize the LCD display

void setup() {

pinMode(buzzerPin, OUTPUT); // set the buzzer pin as output

pinMode(ledPin, OUTPUT); // set the LED pin as output

lcd.begin(16, 2); // initialize the LCD display

analogWrite(6,contrast); // set the LCD contrast

Serial.begin(9600); // initialize the serial monitor

}

void loop() {

// read the values from the microphones

int micValue1 = analogRead(micPin1);

int micValue2 = analogRead(micPin2);

int micValue3 = analogRead(micPin3);

// calculate the sound levels in dB for each microphone

float voltage1 = micValue1 * 5.0 / 1024.0; // convert the first microphone value to voltage (5V reference)

float voltage2 = micValue2 * 5.0 / 1024.0; // convert the second microphone value to voltage (5V reference)

float voltage3 = micValue3 * 5.0 / 1024.0; // convert the third microphone value to voltage (5V reference)

float dB1 = 20 * log10(voltage1/0.0063); // calculate the sound level in dB for the first microphone

float dB2 = 20 * log10(voltage2/0.0063); // calculate the sound level in dB for the second microphone

float dB3 = 20 * log10(voltage3/0.0063); // calculate the sound level in dB for the third microphone

// calculate the average sound level in dB for all microphones

float averageDB = (dB1 + dB2 + dB3) / 3;

// display the sound level on the LCD display and the serial monitor

lcd.setCursor(0, 0); // set the cursor to the first row of the LCD display

lcd.print("Sound Level: "); // print the text "Sound Level: " on the LCD display

lcd.setCursor(0, 1); // set the cursor to the second row of the LCD display

lcd.print(averageDB); // print the average sound level on the LCD display

Serial.print("Sound Level: "); // print the text "Sound Level: " on the serial monitor

Serial.println(averageDB); // print the average sound level on the serial monitor

// control the LED and the buzzer based on the sound level

if (averageDB > 70) { // if the sound level is higher than 70 dB

digitalWrite(ledPin, HIGH); // turn the LED on

tone(buzzerPin, 1000, 500); // turn the buzzer on

} else { // if the sound level is lower than 70 dB

digitalWrite;

}

}

Output:

Sound Level: 52.14

Sound Level: 51.43

Sound Level: 51.41

Sound Level: 51.50

Sound Level: 51.45

Sound Level: 51.70

Sound Level: 51.80

Sound Level: 51.97

Sound Level: 52.36

Sound Level: 52.16

Sound Level: 52.24

Sound Level: 52.27

Web Development:

The link for web development to display real time noise level data is

https://noisepollutionmonitoring1.mydurable.com